Topcon Link

Data Down Loading Process :( Topcon Total Station)

1st Step at computer

Open Topcon link on you start menu.

Click Add New Station to create your station.

Enter name of your new station

Note-

(First Install the driver of data cable, and select the concern port of cable)

Note: (To find port name right click at computer icon, then press manage, then press device manager and click at ports. Prolific USB to serial Comm Port will appear with its port name)

Port-Com1,2,3,4( Select through its concern port of Cable)

Model; GTS220 (only select the model according to series of it)

| OK * | Cancel |
| --- | --- |

New Station:

File Txt.

| FILE* | EDIT | VIEW | PROCESS | WINDOW | HELP |
| --- | --- | --- | --- | --- | --- |

=Import from device

=Station Name

=File Txt

Click >>

Cancel /Start*

Waiting for start. (Before start allocate place to save your data).

(2nd Step at Total Station)

Press Menu on keypad of Total Station, before that make sure the power is on.

Respectively press F3 i.e. Memory Mgr

Than follow the option and press F1 Data Transfer to transfer the data, to move option upward and downward press F4

Than press button F1 i.e. GTS Format, to send data press F1

Than follow the option respectively and than press F1, F2 or F3 for downloading the concern data from your Topcon total station.